Routine PrP IHC staining work sheet – Sequenza and Bench

Purpose of run

# CONJUGATE

# ABC @ 1/25

# DAB

1/25

# LINK

DILUTION(S)

RB V RT @ 1/200

LINK BLOCK(S)

## DILUTION(S)

NRB @ 1/66

# IHC Run No

# ANTIBODY(S)

DILUTION(S)

R145 @ 1/150

BLOCK

DILUTION(S)

NRB @ 1/66

## CONTROL SERA

## DILUTION(S)

##### Initial at each stage 		                                 Time          Initials       Date        Variation            Comments

| 1 | Melt on @ 60°C Yes No | 30 mins |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Dewax, rehydrate and tap water wash @ 19 - 22°C |  |  |  |  | Varistain checked |
| 3 | Epitope unmasking - Formic acid | 30 mins |  |  |  | Fume cupboard |
| 4 | Running tap water | 15 mins |  |  |  | Fume cupboard |
| 5 | Purified water x 2 rinses |  |  |  |  |  |
| 6 | Epitope unmasking - Citrated Autoclaving Total time from heating up to cooling down | 30 mins + 2.5 hours |  |  |  | 121°- 122°C Autoclave No |
| 7 | Running tap water | 10 mins |  |  |  |  |
| 8 | Endogenous peroxidase block – 3% Hydrogen Peroxide (100 vols) in Methanol | 20 mins |  |  |  |  |
| 9 | Running tap water | 10 mins |  |  |  |  |
| 10 | Purified water Room temp. | Overnight |  |  |  |  |
| 11 | TBST wash | 5 mins |  |  |  |  |
| 12 | Normal Serum Block - | 20 mins |  |  |  | Incubate @ °C |
| 13 | 1° Antibody - | 1 hour |  |  |  | Incubate @ °C |
| 14 | TBST wash | 3 x 5 mins |  |  |  |  |
| 15 | Link - | 30 mins |  |  |  | Incubate @ °C |
| 16 | TBST wash | 3 x 5 mins |  |  |  |  |
| 17 | Conjugate - Vector Elite ABC | 30 mins |  |  |  | Incubate @ °C |
| 18 | TBST wash | 3 x 5 mins |  |  |  |  |
| 19 | DABT check |  |  |  |  |  |
| 20 | DABT | 10 mins |  |  |  | Incubate @ °C |
| 21 | Purified water | 5 mins |  |  |  |  |
| 22 | Running Tap water | 5 mins |  |  |  |  |
| 23 | Mayers Haemalum Counterstain | variable |  |  |  | Time |
| 24 | Running Tap water | 10 mins |  |  |  |  |
| 25 | DCM |  |  |  |  | Varistain & Consul checked |
| 26 | Dry |  |  |  |  | Fume cupboard |
| 27 | Scrape & label |  |  |  |  |  |

Total number of slides =

Comments

PAT1 373 v 11 (Implementation date 12/05/2021)								Page 1 of 2

PAT1 373 v9  03.09.10

1 of 2

PAT1 373  Version 7 October   2008									                        1 of  2

IHC Run No 044/23

| ### Pos. No. | Slide Ref HR | Sub No | Desp date/sig/to | Pos. No. | # Slide Ref HR | ### Sub No | # Desp date/sig/to | Pos. No. | Slide Ref HR | Sub no | Desp date/sig/to |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | 41 |  |  |  | 81 |  |  |  |
| 2 |  |  |  | 42 |  |  |  | 82 |  |  |  |
| 3 |  |  |  | 43 |  |  |  | 83 |  |  |  |
| 4 |  |  |  | 44 |  |  |  | 84 |  |  |  |
| 5 |  |  |  | 45 |  |  |  | 85 |  |  |  |
| 6 |  |  |  | 46 |  |  |  | 86 |  |  |  |
| 7 |  |  |  | 47 |  |  |  | 87 |  |  |  |
| 8 |  |  |  | 48 |  |  |  | 88 |  |  |  |
| 9 |  |  |  | 49 |  |  |  | 89 |  |  |  |
| 10 |  |  |  | 50 |  |  |  | 90 |  |  |  |
| 11 |  |  |  | 51 |  |  |  | 91 |  |  |  |
| 12 |  |  |  | 52 |  |  |  | 92 |  |  |  |
| 13 |  |  |  | 53 |  |  |  | 93 |  |  |  |
| 14 |  |  |  | 54 |  |  |  | 94 |  |  |  |
| 15 |  |  |  | 55 |  |  |  | 95 |  |  |  |
| 16 |  |  |  | 56 |  |  |  | 96 |  |  |  |
| 17 |  |  |  | 57 |  |  |  | 97 |  |  |  |
| 18 |  |  |  | 58 |  |  |  | 98 |  |  |  |
| 19 |  |  |  | 59 |  |  |  | 99 |  |  |  |
| 20 |  |  |  | 60 |  |  |  | 100 |  |  |  |
| 21 |  |  |  | 61 |  |  |  | 101 |  |  |  |
| 22 |  |  |  | 62 |  |  |  | 102 |  |  |  |
| 23 |  |  |  | 63 |  |  |  | 103 |  |  |  |
| 24 |  |  |  | 64 |  |  |  | 104 |  |  |  |
| 25 |  |  |  | 65 |  |  |  | 105 |  |  |  |
| 26 |  |  |  | 66 |  |  |  | 106 |  |  |  |
| 27 |  |  |  | 67 |  |  |  | 107 |  |  |  |
| 28 |  |  |  | 68 |  |  |  | 108 |  |  |  |
| 29 |  |  |  | 69 |  |  |  | 109 |  |  |  |
| 30 |  |  |  | 70 |  |  |  | 110 |  |  |  |
| 31 |  |  |  | 71 |  |  |  | 111 |  |  |  |
| 32 |  |  |  | 72 |  |  |  | 112 |  |  |  |
| 33 |  |  |  | 73 |  |  |  | 113 |  |  |  |
| 34 |  |  |  | 74 |  |  |  | 114 |  |  |  |
| 35 |  |  |  | 75 |  |  |  | 115 |  |  |  |
| 36 |  |  |  | 76 |  |  |  | 116 |  |  |  |
| 37 |  |  |  | 77 |  |  |  | 117 |  |  |  |
| 38 |  |  |  | 78 |  |  |  | 118 |  |  |  |
| 39 |  |  |  | 79 |  |  |  | 119 |  |  |  |
| 40 |  |  |  | 80 |  |  |  | 120 |  |  |  |

## Controls / Quality Check

| Slide No | Section No | Hist Ref HR | TSE status | Tissue | #### Diagnostic Pass/Fail P/F | #### Research Increase/ Decrease/ No Change | Date/ Sign/ to | Comments Include non-specific staining |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

PAT1 373 v 11 (Implementation date 12/05/2021)								          Page 2 of 2

PAT 1 323    Version 1 3 June 2004                                                                       Page 2